#R1

Q1: If the argument against vapor cell includes mm-thick walls for limiting short-range interactions below 100 um, I am not understanding how cold-atom comagnetometers over come this range in part by not having ‘cell walls’---isn’t the cold-atom comagnetometer contained within a vacuum chamber?

A1: For the detection of short-range interactions, it is necessary to vary the distance between the test mass and the atoms. In a thermal atom system, the test mass is placed outside the vapor cell to change the separation, so the minimum distance is limited by the cell wall thickness, typically on the millimeter scale. In contrast, in a cold atom system, atoms are confined in an optical trap with a radial size generally between 1 and 100 μm. Here, both the optical trap and the test mass are enclosed within a vacuum chamber, and the test mass can be repositioned within the chamber. With no optical viewports or chamber walls between the test mass and the atoms,​ their separation can be reduced to below 100 μm, even to the micrometer scale.

在短程相互作用的探测中，需要调节测试质量块和原子的间距。若采用热原子方法，质量块需置于vapor cell外，以改变距离，因此热原子与质量块的最短间距不能小于cell壁，其一般在mm量级。而若采用冷原子方法，原子由光阱囚禁，其径向尺寸一般在1-100 μm，光阱和质量块都封装在真空腔内，质量块位置可调，二者之间没有窗片和真空腔壁阻隔，原子和质量块的距离可以低于100 μm，甚至低至μm尺度。

Q2: The total field used herein is relatively high compared to some of the other paramagnetic/paramagnetic-diamagnetic/and diag\magnetic pair systems you’ve mentioned in the introduction, which you allude to in the introduction. Doesn’t this affect your absolute energy resolution quite significantly as compared to these systems, as a ppm on 0.1 G is not as impressive as a ppm on the much lower fields used in those studies?

A2: For the single-component 171Yb magnetometer, with a precession time of 10 s under a 0.1 G field, the single-shot absolute energy (frequency) sensitivity is 2π×2 mHz. This sensitivity is primarily limited by atomic spin-projection noise and the residual light shift introduced by the optical trap, while the noise contributed by the 0.1 G magnetic field plays a minor role.

若仅讨论171Yb单组分磁力仪，在0.1 G磁场下进动10 s，单次探测绝对能量（频率）灵敏度为2π×2 mHz，其受限于原子自旋投影噪声，和光阱引入的剩余light shift，而0.1 G磁场引入的噪声对灵敏度的影响程度较小。

For the comagnetometer, considering a suppression factor of 3×104, magnetic field noise is further reduced to a negligible level, while the sensitivity to weak signals of interest such as new‑physics interactions remains unaffected. The single-shot sensitivity is maintained at 2π×2 mHz. Accumulating data over 24 hours (~3000 measurements) yields a sensitivity of 2π×40 μHz. The following response compares this result with other relevant work.

再讨论共磁力仪，考虑到3×104的抑制比，磁场噪声被进一步压制，其影响可忽略。而对于共磁力仪所探测的微弱信号例如新物理相互作用，其灵敏度并不受抑制比压制，单次探测的灵敏度仍为2π×2 mHz，若累积24 h数据（~3000次测量），灵敏度约为2π×40 μHz。在下一个问题的回答中，我们对此和其它相关的工作进行了比较。

Q3: Related to 2, how does your ppm performance and absolute energy resolution compare to other trapped atom systems more similar to your own?

A3: Currently, there are two related works based on trapped atom systems. One utilizes two hyperfine levels of 87Rb (Gomez, P. et al., 2020 [24]) with oppositely signed electron-spin magnetic moments. Adding the precession signals from these two levels suppresses common-mode electron‑spin magnetic noise, which allows the nuclear‑spin precession signal to be extracted. As a result, the system effectively probes the nuclear spin rather than the electron spin, and the coherence time is extended from milliseconds to about 1 s. This method essentially implements a single magnetometer based on nuclear spin, not a comagnetometer. In contrast, our comagnetometer measures the Larmor-frequency ratio of two different nuclear spins, which is a dimensionless quantity insensitive to magnetic‑field fluctuations. Consequently, the two approaches are fundamentally different, both in the measured physical quantity and in the underlying noise-suppression principle.

当前有两项基于trapped atom systems的工作。其中一项工作使用了87Rb的两个超精细能级（Gomez, P. et al., 2020 [24]），其电子自旋磁矩符号相反。将这两个能级的进动信号相加，可以抑制共模的电子自旋磁噪声，从而提取出核自旋的进动信号。这使得系统的有效探测对象由电子自旋变为核自旋，相干时间随之从ms量级提升至1 s。该方法本质上构建了单个基于核自旋的磁力仪，而非共磁力仪。而我们的共磁力仪的测量对象为两种不同核自旋的Larmor频率比，是一个对磁场涨落不敏感的无量纲值。两项工作的测量物理量和噪声抑制原理存在根本不同。

The other work implements a comagnetometer with 87Rb and 87Sr (Thekkeppatt, P. et al., 2025 [25]), where 87Rb provides magnetic‑field correction. In order to suppress rapid decoherence of 87Sr induced by the optical‑trap light shift, a magnetic field of 100–1000 G is applied. This, however, limits the 87Rb coherence time to millisecond scale and constrains the absolute energy sensitivity. As reported, with 1.5 h of integration, the frequency‑ratio measurement attains a precision of 1.4 ppm, and the absolute energy (frequency) sensitivity for 87Sr is approximately 2π×0.12 Hz. This ratio precision is limited by systematic errors and remains unimproved with longer integration time, to which our 4 ppm result is comparable.​ Extending the integration to 24 h would improve the absolute energy sensitivity of 87Sr to 2π×25 mHz. For detecting weak signals such as new‑physics interactions, our energy sensitivity of 2π×40 μHz at 24 h represents a clear advantage.

另一项工作由87Rb和87Sr组成共磁力仪（Thekkeppatt, P. et al., 2025 [25]），其中87Rb用于磁场校正，为了避免87Sr受光阱引入的light shift影响导致迅速退相干，他们需要施加100-1000 G的工作磁场，但也因此导致87Rb仅有毫秒级别相干时间，并限制了绝对能量灵敏度。按文章所述，累积1.5 h数据，频率比测量精度可达1.4 ppm，87Sr的绝对能量（频率）灵敏度约为2π×0.12 Hz。1.4 ppm频率比测量精度受限于系统误差，不因测量时间累积提高，与我们的4 ppm相当。若累积24 h数据，87Sr的绝对能量（频率）灵敏度约为2π×25 mHz。因此在测量微弱信号例如新物理相互作用方面，我们的能量灵敏度(2π×40 μHz @ 24 h)存在明显优势。

Q4: The proposals at the end of the paper involving EDM searches, including those that incorporate 223Ra are 225Ra are ambitious, but I wonder how much these actually have to do with your current manuscript? You give no hard estimates of improvements sensitivity of your proposals herein, so it does not make sense to me to include them here.

A4: In response to the reviewers' comments, we have removed the Ra-related material.

我们已听取审稿人意见，将Ra相关内容删除。

#R2

Q1: The "cat" state used for Yb-173, is it what is called a NooN state? Should the connection be mentioned?

A1: The NOON state, which takes the form N,0+0,N, describes a coherent superposition of all N particles occupying one quantum state, and all occupying another orthogonal state. The cat state of atomic spin F,+F+F,-F exhibits an analogous structure in Hilbert space. Following the reviewer's suggestion, we have added a corresponding mention and citation (Wolfgramm, F. et al., 2010 [26]) in the main text.

NOON态的形式为N,0+0,N，描述N个全同粒子完全占据某一量子态与完全占据另一正交态的相干叠加。原子自旋猫态F,+F+F,-F在希尔伯特空间中具有类似NOON态的结构。我们已按评审提议，在正文中增添提及并引用相关文章 (Wolfgramm, F. et al., 2010 [26])。

Q2: In Fig. 4, there are significant deviations of the experiment from the model, why?

A2: Our previous simulation was oversimplified. It only accounted for the detection of atoms in the mF=±5/2 states. In measuring this curve, we actually prepared​ the 173Yb ensemble in a coherent spin state encompassing all mF substates, not a cat state, and the detection cannot adequately resolve all these individual substates. Additionally, the previous simulation assumed a single, fixed tensor light shift. In reality, the tensor shift varies with position in the optical trap, and the overall signal represents an average. Consequently, the previous theoretical prediction exhibited sharp peaks absent from the experimental data.

此前我们的simulation过于简化，仅考虑了探测到mF=±52的可能，实际在测量这条曲线时，我们将173Yb制备在包含所有mF态的coherent spin state，而非cat state。探测时，并不能对所有mF态做出较好分辨。另外，此前假设tensor shift为单一固定值，实际不同原子在光阱不同位置感受到的tensor shift也不同，整体展现为平均效应。综上，此前的理论曲线出现了与实际测量值无法贴合的尖峰。

In response to the reviewer’s comments, we have revised the simulation model. With the updated model incorporating the detuning of the probe beam from each mF transition and the corresponding Clebsch–Gordan coefficients, we calculate the overall detection signal from all mF states. Specifically, when probing the |mF=+5/2⟩ state, the detection probabilities for the mF states (from +5/2 to −5/2) are 1, 0.58, 0.31, 0.16, 0.08, and 0.03. We further include the distribution of the tensor light shift experienced by atoms in the optical trap. After re-running the simulation, the sharp features in the theoretical curve are smoothed, leading to improved agreement with the experimental data. The simulation details are provided in the Methods section, under “Calibration of ΩB/ΩT”.

根据评审人意见，我们对simulation模型进行了改进。在改进的模型中，我们考虑了探测光对173Yb各mF跃迁的失谐，并结合相应的Clebsch-Gordan系数，计算了探测信号对所有mF态的整体响应。具体来说，当探测对象为|mF=+52，探测到各mF态的概率（从+5/2到-5/2）分别为：1, 0.58, 0.31, 0.16, 0.08, 0.03。再考虑到原子在光阱中感受到的tensor场的分布，我们重新进行simulation，理论曲线的尖峰因此变得平滑，与实验值更贴合。具体simulation 细节已放入method section “Calibration of ΩBΩT”。

We also note that elsewhere in the paper, 173Yb are prepared in cat state only involving |mF=±5/2⟩ sublevels, which exhibit the maximally different detection probabilities. Consequently, these two mF sublevels can be clearly resolved, and the aforementioned detection issue does not arise. Details are provided in the Methods section “Atom sample detection.”

另外需说明的是，在文章其它部分，173Yb被制备到cat state，仅包含探测概率相差最大的|mF=±52两个子态，因此可清晰分辨这两个mF子态，未出现上述探测问题。细节详见method section “Atom sample detection”。

Q3: Please check the formatting of the references. I noticed capitalization problems in Refs. 19 and 26.

A3: In accordance with Nature’s style guidelines, we have revised the manuscript throughout and corrected the capitalization errors—including “Yb” in the original Ref. 26, as well as similar errors in Ref. 23 and Ref. 24.

我们已按照nature惯用格式要求，进行了全文检查修改，修正了包括原Ref. 26的”Yb”以及Ref. 23, Ref. 24等多处大小写错误。

Q4: The authors mention the simultaneous effect of the Zeeman and tensor light shifts. In fact, this is a major effect (often referred to alignment-to-orientation conversion) that plays an important role in nonlinear Faraday rotation. The authors might wish to make this connection, if they think it is relevant.

A4: The alignment-to-orientation conversion (AOC) in nonlinear Faraday rotation stems from the tensor light shift of probe beam, which induces a coherent evolution of spin states (Budker, D. et al., 2000 [30]). By applying this scheme, we drive the atoms from an initial orientation state to an alignment state, thereby preparing the cat state.

在nonlinear Faraday rotation中，alignment-to-orientation conversion (AOC)来源于探测光的tensor shift，是自旋态在tensor shift下的相干演化 (Budker, D. et al., 2000 [30])。我们猫态制备正是利用了这一原理，让原子由最初的orientation state变为alignment state。

Furthermore, the phenomena are also interconnected in the simultaneous effect of the Zeeman and tensor light shifts. In our experiment, when the laser polarization is perfectly aligned with the magnetic field, the alignment state (i.e., the cat state) constitutes an eigenstate of the tensor light shift, thereby rendering the phase immune. However, if there is a misalignment between the laser polarization and the magnetic field, cat state experiences a twist due to the tensor shift, as described by AOC. Therefore, we have addressed the connection in section “Tensor shift” and added the corresponding citation (Budker, D. et al., 2000 [30]).

此外，在simultaneous effect of the Zeeman and tensor light shifts中，两种现象也有关联。我们的实验中，当激光偏振完美平行于外磁场方向时，alignment state (cat state)同时也是tensor light shift的本征态，cat state相位不受tensor shift影响；但当激光偏振与外磁场存在角度时，cat state受tensor shift影响发生twist，这正是AOC所描述的效应。因此我们在section Tensor shift中提及了两者的关联，并增加了引用 (Budker, D. et al., 2000 [30])。

#R3

The biggest achievement of the paper is, to us, the implementation of a method to suppress light shifts induced by the trapping lattice, and the subsequent measurement “with a precision of 4 ppm” of the magnetic moment ratio between 171Yb and 173Yb.

Q1a: How valuable is this measurement/Why is it important?

A1a: The nuclear magnetic moment is a fundamental property of the nucleus. The ratio of nuclear magnetic moments between different isotopes reveals the relative evolution of their nuclear structures. The established method for determining an unknown nuclear magnetic moment is to measure its ratio to a known reference moment. Therefore, improving the precision of these ratio measurements is crucial for​ enhancing the overall precision of nuclear magnetic moment determinations.

核磁矩是核的基本性质，核磁矩之比反映了不同同位素间核结构的相对演化。当前测量核磁矩的主流方法是选定某一已知磁矩作为参考，测量其与未知磁矩之比，由此得到未知磁矩值，因此提升核磁矩比的测量精度有助于普遍提升核磁矩的测量精度。

Q1b: Why not provide the measured value directly in the abstract?

A1b: In response to the reviewer's suggestion, we have added the measured value to the abstract.

应评审人建议，我们已将测量值加入摘要。

Q1c: Have there been more measurements of this ratio apart from these of Refs. [29] and [30]? If so, a summary plot showing each measured value and associated uncertainty would be welcome. Are all the quoted values (authors and previous works) given with a 1-σ uncertainty?

A1c: There are two prior sets of measurement results. As suggested by the reviewer, we have compiled a comparative data table in the main text (values in parentheses are 1-σ uncertainties). Since the error bars of the different datasets differ by orders of magnitude, it is impractical to present in a single plot with a common scale. Therefore, we present the comparison in tabular form.

此前有两项测量结果，根据审稿人的建议，我们已在正文中补充了数据对比表格（所有括号内值均为1-σ uncertainty）。由于各组数据的errorbar量级差异过大，若采用绘图呈现，难以在同一坐标尺度内清晰展示，因此我们选用了表格形式。

Q1d: In the part Ratio of nuclear magnetic moments (page 8), the authors compile values of the ratio of nuclear magnetic moment, RL, measured while different experimental parameters are varied to search for systematic effects (Figs. 2 and 3). Why are the different values not overlapping? Please comment on how the average is weighted and why.

A1d: In Figs. 2b and 2d, RL corresponds to the extremum point obtained from a parabolic fit to the data measured under different magnetic fields (50 mG and 100 mG). This is intended to verify the validity of the tensor shift theory. The fitting results confirm that the two are consistent within the error bars; that is, when the tensor field is aligned with the magnetic field, RL is independent of magnetic field strength.

Fig. 2b和Fig. 2d图中的RL为对不同磁场下(50 mG和100 mG)测得的数据做抛物面拟合得到的极值点，意在验证tensor shift理论的正确性，拟合结果证明二者在误差允许范围内相符，即当tensor场和主磁场平行，RL与磁场大小无关。

After aligning​ the magnetic and tensor fields according to Fig. 2, measurements for both Fig. 3a and 3b were conducted at 100 mG. These figures investigate the dependence on the optical trap power (i.e., the light shift) and on the density ratio of the two isotopes, respectively, with no observable dependence in either case. Fig. 3a and 3b​ each independently yield a weighted mean and an associated error bar. Since these two values are consistent within their respective uncertainties, we took their weighted average to obtain the final ratio of the nuclear magnetic moments. Additional details have been added in the Section “Ratio of nuclear magnetic moments”.

因此，在依据Fig. 2完成主磁场与tensor场对齐后，Fig. 3a和Fig. 3b均在100 mG磁场下测量，分别测量对光阱功率（即light shift），以及对两同位素密度比的依赖，均未观察到依赖关系。Fig. 3a和Fig. 3b各自独立测得一个加权平均值与误差棒，二者在误差允许范围内相符，我们将这两个值加权平均，得到了最终的核磁矩之比。在Section “Ratio of nuclear magnetic moments”中，我们增加了相关的说明细节。

As shown in the table below, the four RL values—namely, the extremum points from Figs. 2b and 2d, and the weighted means from Figs. 3a and 3b, respectively—agree with each other within their error bars. The value from Fig. 2b was obtained under a different main magnetic field and was therefore not included in the final calculation of the nuclear magnetic moment ratio. Meanwhile,​ the extremum point from Fig. 2d, the 30 W Lattice beam power point in Fig. 3a, and the second data point from the left in Fig. 3b were all obtained under identical experimental settings.

由下方表格可见，四个RL值（分别来自Fig. 2b, Fig. 2d的极值点，以及Fig. 3a和Fig. 3b各自的平均值）在误差范围内彼此符合。其中，Fig. 2b因所用主磁场条件与其他测量不同，未被纳入最终核磁矩比值的计算；而Fig. 2d的极值点，Fig. 3a中光阱功率30 W对应的点，以及Fig. 3b中从左至右第二个点，是在相同实验设定条件下获得的。

|  | RL |
| --- | --- |
| Fig. 2b | 0.726079(3) |
| Fig. 2d | 0.726080(5) |
| Fig. 3a | 0.726077(2) |
| Fig. 3b | 0.726072(4) |
| Final value | 0.726076(3) |

The perspective of the paper concerns the search the of EDM using their experimental apparatus. As we understand, the main point here is that using a comagnetometer (rather than a single specie) should help reduced magnetic field noises which are currently the limiting factor.

Q2a: In the introduction, authors claim that the glass vapor cell comagnetometer are unsuitable to search for “short-range interactions below ~ 100 µm”. Could they elaborate? A citation would help here.

A2a: In Feng, Y.-K. et al., 2022 [13], the authors note that, “its (the cell) end wall perpendicular to the long axis has a thickness of 0.5 mm”, and “Current results are mainly limited by the Rb magnetometer sensitivity, and by the minimum distance between the BGO mass and Xe atoms. ” In these vapor-cell experiments, the thickness of the cell walls prevents the atom–mass separation from sub-100 μm range. Moreover, atoms are distributed throughout the cell and rarely reside closest to the mass, which dilutes the short‑range effect.

在以上vapor cell实验中，腔壁的厚度会限制原子和质量的间距难以进入sub-100 μm 范围。另，由于原子分布在整个cell内，原子大部分时间并不处于距离质量块最近的位置，因此稀释了短距离效应。

Q2b: Authors estimate an improvement of the sensitivity to spin-mass coupling by up to three orders of magnitude. Could they also elaborate on this? A plot showing current limits together with their projected sensitivity would, in my opinion, strengthen the paper.

A2b: The green curve (5) shows the projected sensitivity for a co‑magnetometer consisting of a single ¹⁷¹Yb and a single ¹⁷³Yb atom in the tweezer, with a platinum mass source placed 5 μm away. Assuming a measurement cycle of 30 s (10 s interrogation time) and a total integration time of 30 days (~30,000 cycles), the projection indicates that the sensitivity to gsgp at λ=4 μm would improve by three orders of magnitude over the previous best limit (Guigue, M. et al., 2015 [36]). As suggested by the reviewer, this description has been added to the main text.

下图绿色曲线5是我们预估的灵敏度曲线。此处考虑tweezer中由单个171原子和单个173组成的共磁力仪，质量块采用铂，与原子的最近距离设置为5 μm。假设单次测量时间 30 s，其中interrogation time 10 s，共计累积30天数据（~30000组）。由图中可见，在λ=4 μm处，对gsgp的灵敏度能够比此前最好结果(Guigue, M. et al., 2015 [36])提升3个量级。根据评审意见，加了文字描述。

Currently, the experimental apparatus is under construction. The specific physical parameters involved remain to be determined, and the relevant systematic errors require experimental characterization. Consequently, we have opted not to include a projected sensitivity curve for the follow-up work at this stage.

当前我们正在搭建实验装置，以上物理参数尚未确定，相关系统误差仍需通过实验探索，因此我们暂不发表后续工作中的灵敏度曲线图。

Figure 1. Constraints on the coupling constants gsgp of the spin-dependent force as a function of the force range λ. Line 1, 3He depolarization: Guigue, M. et al., 2015. Line 2,  129Xe− 131Xe−Rb comagnetometer: Feng, Y.-K.et al., 2022. Line 3,  129Xe− 131Xe−Rb comagnetometer: Bulatowicz, M. et al., 2013. Line 4,  3He− 129Xe comagnetometer: Tullney, K. et al., 2013. Line 5: The predicted results of this work.

Q3:“The ratio of their Larmor frequencies —termed the comagnetometer ratio—” We are not sure that this is a standard term.

A3: The term "comagnetometer ratio" was introduced for convenience and is not a standard term. It has been changed to "Larmor-frequency ratio" throughout the manuscript.

Comagnetometer ratio是我们为方便起见自行定义的名称，并非标准说法，文章中已修改为Larmor-frequency ratio.

Q4: In the introduction, the authors mention that they have identified a “novel systematic effect stemming from the interplay between Zeeman and tensor light shifts”. Does this refer to the discussion in section “Tensor shift”?

A4: Yes. The Larmor-frequency ratio is affected differently depending on the relative strength between the magnetic field and the tensor field.

是的。当磁场和tensor场在不同比例下，对频率比也会体现出不同的影响。