Cold-atom comagnetometry via optical

control of spin states

J.-L. Zhang1t,  W.-T. Luo1t,  Y. A. Yang1 ,  Y.-Q. Wang1 ,

T. Xia2*,  Z.-T. Lu1,2*

1 Hefei National Research Center for Physical Sciences at the

Microscale, School of Physical Sciences, University of Science and

Technology of China, Hefei, Anhui, 230026, China.

2 Hefei National Laboratory, University of Science and Technology

of China, Hefei, Anhui, 230088, China.

*Corresponding author(s). E-mail(s): txia@hfnl.cn; ztlu@ustc.edu.cn;

Contributing authors: jielin@mail.ustc.edu.cn;

luowt@mail.ustc.edu.cn; yyustcer@mail.ustc.edu.cn;

wyq0705@mail.ustc.edu.cn;

t These authors contributed equally to this work.

Abstract

Atomic  spin-based  comagnetometers  are  powerful  tools  for  precision sensing  and  tests  of  fundamental  physics.  Compared  with  the  widely used gas-cell comagnetometer systems, cold-atom systems offer access to much shorter distance scales and allow implementation of optical quan- tum control techniques. However, in order to realize long spin coherence times with cold atoms, it is necessary to employ diamagnetic atoms and overcome  decoherence  induced  by  light  shifts.  Here  we  demonstrate  a cold-atom comagnetometer based on the nuclear spins of 171Yb  (spin- 1/2) and 173Yb (spin-5/2), jointly trapped in an optical lattice. Vector light shifts are suppressed by enforcing linear polarization of the lattice, while tensor shifts in 173Yb are suppressed via the use of a Schro…dinger cat state. This enables simultaneous Ramsey interferometry on both iso- topes with a spin coherence time of 60 s. We achieve a magnetic noise suppression factor exceeding 3 × 104 , and determine the ratio of nuclear magnetic moments as -0.726076(3), with a relative uncertainty of 4 ppm.

Our  results  establish  a  new  cold-atom  platform  for  spin-based  sens- ing and open pathways toward quantum-enhanced searches for physics beyond the Standard Model.

Keywords: optical quantum control, Ytterbium, comagnetometer,

suppression of magnetic noise, spin sensor, nuclear magnetic moment

# Introduction

Atomic-spin-based sensors are indispensable in quantum information science [1] and precision measurement [2, 3]. Paramagnetic atoms, whose spins carry Bohr magnetons, underpin some of the most sensitive magnetometers [4]. In contrast, diamagnetic atoms possess spins originating solely from their nuclei. These nuclear spins, governed by much smaller nuclear magnetons, exhibit significantly longer coherence times [5], making them ideal for sensing interac- tions beyond magnetism. Nuclear magnetic resonance (NMR) is a prominent example of such sensing.

In order to suppress magnetic-field noise, a comagnetometer employs two atomic  species  with  distinct  gyromagnetic  ratios,  colocated  and  measured simultaneously [6]. The ratio of their Larmor frequencies is intrinsically insen- sitive to magnetic-field fluctuations while retaining sensitivity to nonmagnetic interactions. Such systems have demonstrated the exceptional ability to detect energy shifts at the pHz level [7].

Room-temperature gas-cell comagnetometers have been extensively devel- oped,  with  combinations  including  the  85 Rb-87 Rb  paramagnetic  pair   [8], paramagnetic-diamagnetic  pairs  K-3 He   [7]  and  Rb-21 Ne   [9],  and  diamag- netic  pairs  3 He-129Xe   [9—11],  Xe  isotopes   [12, 13]  and  Hg  isotopes  [14]. These platforms support a wide range of applications, from inertial sensing [15] to precision tests of fundamental physics, including searches for electric dipole moments  (EDMs)  [16—18],  monopole—dipole interactions  [12, 13, 19], spin—gravity  couplings  [7, 8, 20, 21],  and  axion-like  dark  matter  [22,  23]. However, their centimeter-scale cell sizes and millimeter-thick walls impose a  limitation  on  sensitivity  to  short-range  interactions  with  ranges  below ~ 100 μm.

Cold-atom comagnetometers can overcome this limitation. Atom clouds confined in a laser trap can be reduced to scales of 1—100 μm, enabling sen- sitivity to comparable interaction ranges. Moreover, the absence of material walls eliminates related systematic effects. A prior implementation used two hyperfine states of paramagnetic 87 Rb with spin coherence times at several milliseconds [24]. Signals of the two hyperfine states were combined to gener- ate a single-species magnetometer based on the nuclear spin with a coherence time of 1 s. More recently, a cold-atom system combining paramagnetic 87 Rb with diamagnetic 87 Sr was realized [25]. Operating under magnetic fields of

100-1000 G, this system enabled ppm-level determination of the nuclear g fac- tor of 87 Sr, though its coherence time was ultimately limited to milliseconds by the 87 Rb component under strong magnetic fields.

Here, we realize a comagnetometer based purely on nuclear spins consisting of 171Yb (spin-1/2) and 173Yb (spin-5/2), simultaneously confined in a one- dimensional optical lattice under a weak magnetic field (10-100 mG). A key challenge in such systems is the suppression of lattice-induced light shifts. We eliminate vector light shifts by enforcing linear polarization of the lattice beam. Tensor shifts are naturally absent in 171Yb due to its spin-1/2 character. For the higher-spin 173Yb, we apply quantum control techniques to prepare a  Schr¨odinger cat state, whose precession frequency is rendered insensitive to tensor shifts. Using simultaneous Ramsey interferometry on both isotopes, we measure the Larmor frequency ratio and demonstrate a spin coherence time of 60 s. We identify a novel systematic effect in the cat state stemming from the interplay between Zeeman and tensor light shifts. By understanding and controlling both vector and tensor light shifts, we achieve a magnetic- noise suppression factor exceeding 3 × 104 . We determine the ratio of nuclear magnetic moments between 171Yb and 173Yb with a precision of 4 ppm.

# Realization of cold-atom comagnetometer

In preparation of a cold-atom ensemble, ytterbium atoms containing both 171Yb  and  173Yb  (isotopic  abundances:  14%  and  16%)  exit  an  oven,  pass through a Zeeman slower, and are captured by a blue magneto-optical trap (MOT) formed with a laser exciting the broad transition 6s2   1 S0  → 6s6p 1 P1 at 399 nm. The atoms are subsequently cooled to a temperature of  ~ 20 µK with a green MOT at 556 nm acting on the narrow transition 6s2   1 S0   → 6s6p 3 P1 , and then transferred to a movable optical dipole trap (Bus ODT) at 1036.4 nm, which is a magic wavelength for the cooling transition [26]. Carried by the Bus ODT from the MOT chamber to an adjacent science chamber across 0.35 m, the atoms arrive at their final destination: a horizontally directed, one- dimensional optical lattice also at 1036.4 nm (Fig. 1a). With a beam power of 30 W and a waist of 25 µm, the lattice consists of trap sites with a depth of ~ 2 mK. A total of about 2 × 104   171Yb atoms and 2 × 104   173Yb atoms are mixed together to form an ellipsoidal cloud with a diameter of 40 µm and a length of 80 µm, occupying approximately 160 lattice sites. The lifetime of atoms in the optical lattice is observed to be 50 s.

The science chamber is surrounded by a set of coils and magnetic shields. A uniform magnetic field of 10 -  100  mG in the  Z-direction is applied to define the spin quantization axis (Fig. 1a). In the 1 S0 , F = 1/2 ground state of 171Yb,  a transversely  polarized  state  |ψt〉 =  |F = 1/2, mF  = +1/2〉g   + eiϕ1  |F = 1/2, mF  = −1/2〉g  is employed to sense the precession phase ϕ 1 . For the ground state of 173Yb (I = 5/2), it is necessary to prepare atoms into the cat state, |ψc〉= |5/2, +5/2〉g  + eiϕ2   |5/2, −5/2〉g , in order to achieve immu- nity to otherwise significant decoherence caused by tensor light shifts in the

optical lattice [27]. The cat state cannot simply be prepared with either opti- cal pumping or a rotating magnetic field. Instead, it is generated using a 556 nm Rabi beam, a circularly polarized laser beam traveling in the X-direction to drive the necessary Rabi transitions, with its ~ GHz detunings from the 6s2  1 S0  → 6s6p 3 P1  transitions designed to induce a specific combination of vector and tensor light shifts [27]. For convenience, the same Rabi beam is also used to drive Rabi transitions in 171Yb, where it induces a pure vector shift in the spin-1/2 system. In this work, the induced Rabi transition frequency is 2π × 0.4 kHz for 173Yb, and 2π × 3 kHz for 171Yb.

Ramsey interferometry is simultaneously implemented on both isotopes. Its measurement sequence is illustrated in Fig. 1b. Since all laser pulses are simul- taneously shone on atoms of both isotopes, care needs to be taken either to avoid crosstalks between the two isotopes, or to compensate for any crosstalk effects. First, optical pumping at 399 nm along the Z-axis is applied to polar- ize 173Yb and 171Yb successively. Then, a π/2-pulse of the 556 nm Rabi beam is applied to drive the spin state of 171Yb. The pulse also induces a small rotation (~ π/16) on the spin of 173Yb. After a quarter of the 171Yb-Larmor period (T171 /4), a π/2-pulse of the Rabi beam is applied to drive 173Yb into the cat state, while having little effect on 171Yb because their spins are point- ing along the Rabi beam direction at the time of the pulse. Afterwards, spins of both isotopes are allowed to precess freely and accumulate phases over time. At the readout stage, first on 173Yb, the phase information is converted into population values by applying a π/2-pulse to the precessing cat state. The populations are measured as follows (see Methods): the first probe pulse at 556 nm measures the population of |5/2, +5/2〉g , yielding N173+ ; a subsequent π-pulse exchanges the populations between |5/2, +5/2〉g  and |5/2, −5/2〉g ; the second probe then captures the original population of |5/2, −5/2〉g , denoted as N173 - . The differential signal is derived as Sz  = (N+  − N- )/(N+  + N- ). Note that both the second π/2-pulse and the π-pulse on 173Yb unavoidably operate on 171Yb as well. To compensate their effects on 171Yb, an additional pulse is introduced so that the three pulses, each successive one delayed by a complete 171Yb-Larmor period (T171 ), combine to rotate the spin of 171Yb by (12 + 1/2)π . Finally, the populations of 171Yb spin states are read out in a way similar to that of 173Yb.

The resulting Ramsey fringes are displayed in Fig. 1c. The contrast of 173Yb decays due to interspecies collision between  173Yb and 171Yb, reducing the coherence time of 173Yb from thousands of seconds down to 60 s. Meanwhile, the collisions seem to have no degradation effects on 171Yb.

By applying simultaneous Ramsey interferometry on both isotopes, we determine the precession phases, and derived the Larmor frequencies of 171Yb (ω171 ) as well as the cat state of 173Yb (ω173 ), both with a statistical sensitivity of ~  2π × 2 mHz in a single-shot measurement.

Under  ideal  conditions,  the  comagnetometer  frequency  ratio  RL     = ω 171 /ω 173  is insensitive to magnetic-field fluctuation. However, we find that

light shifts induced by the high-power optical lattice could cause a significant perturbation to RL .

a                                                                             c

1

0.5

z

S

0

-0.5

171Yb

173Yb

-1

0                       10                     20                     30                     40

Precession time - 10 s (ms)

b

Fig. 1    171 Yb-173 Yb cold-atom comagnetometer. (a) Atoms of 171 Yb and 173 Yb are mixed and confined together  in the optical  lattice. A  uniform field of  10-100  mG  is applied to define the quantization axis in the vertical Z-direction. While  171Yb  (I =  1/2) naturally has no tensor light shifts, 173Yb (I = 5/2) is prepared into the cat state, which is immune to decoherence caused by the tensor shifts in the optical lattice. (b) Measurement sequence of the simultaneous Ramsey interferometry. The arrangement of the 399 nm pump pulses (solid purple), 556 nm Rabi pulses  (solid dark green), and 556 nm probe pulses  (hatched light green) are explained in details in the main text.  (c) Ramsey signals at  10 s with a bias field at 50 mG, where red circles represent the precession signals of 171Yb, and blue squares correspond to those of 173 Yb. Error bars: one standard deviation (1-σ) derived from 3 repeated measurements; solid curves: least-squares fittings with sinusoidal functions.

# Dependence on light shift

The light shift can be decomposed into scalar, vector, and tensor terms, which are respectively proportional to the zeroth, first, and second powers of the magnetic quantum number mF   [28]. The scalar shift causes identical displace- ments for all mF  states, thereby contributing no phase shifts in the evolution of angular momentum states. The vector shift, induced by circularly polarized light, acts as a Zeeman-like shift with its quantization axis along the wave vec- tor of the light field. Given the linearly polarized optical lattice used in this work, the tensor shift aligned along the polarization direction emerges as the predominant effect.

# Tensor shift

Tensor shifts are naturally absent in the ground state 1 S0  of 171Yb (I = 1/2). For 173Yb (I = 5/2), tensor shifts are also expected to be null in the cat state of 1 S0 , where perfect alignment between the lattice polarization and the magnetic field B would make the alignment state an eigenstate of the tensor shift. How- ever, a small angle θ exists between the polarization (z) and B (along Z). This misalignment twists the alignment state, producing a frequency shift through a mechanism similar to that described by alignment-to-orientation conversion (AOC) [29]. Without loss of generality, assume the magnetic field has a small component in the transverse x-direction, the Hamiltonian H arising from the combined magnetic and tensor fields can be expressed as

H = ΩB (σz  + θσx ) + ΩT σ2z (θ ≪ 1).                          (1)

Here, σz  and σx  are the angular momentum matrices, with their terms corre-

sponding to Zeeman effects. The σ2z term describes the tensor effect. ΩB  and

ΩT  are the corresponding coupling parameters.

Under constant optical trapping conditions, we scan the orientation of the bias magnetic field over two dimensions and map the Larmor frequency ratio RL . First, the magnetic field is set at 50 mG, corresponding to ΩB /ΩT  = 2.2 (see Methods). The measured RL   values fall onto a parabolic surface with a minimum extremum RL,min   =  0.726079(3)  (Fig. 2b).  Next,  at  a higher magnetic field of 100 mG, where ΩB /ΩT  = 4.3, the opening direction of the parabolic surface is found to be inverted, yielding a maximum RL,max   = 0.726080(5)  (Fig.  2d),  with  its  value  consistent  with  the  minimum  value obtained at 50 mG. The parabolic surface and its curvature inversion can be attributed to a second-order perturbation effect on the 173Yb energy levels, with the transverse σx  term treated as a perturbation. The resulting shifts on the energy splitting between |5/2, +5/2〉g  and |5/2, —5/2〉g  can be expressed as

where δEF)  denotes the 2nd-order perturbation correction to the energy level

of |5/2, mF〉g . Derivation of Eq. (2) is presented in Methods. It can be seen that, on opposite sides of the critical ratio ΩB /ΩT  = 4, the sign of ∆E reverses, resulting in curvature inversion of the RL   surface. At the critical ratio of ΩB /ΩT   = 4, due to the degeneracy between  |5/2, —5/2〉g   and  |5/2, —3/2〉g , the above perturbation treatment is no longer valid, and the cat state can no longer be preserved.

In order to evaluate the robustness of the comagnetometer against envi- ronmental  magnetic  field  fluctuations,  we  define  the  suppression  factor  η as:

From 50 mG to 100 mG, the magnetic field changes by a factor of two, so are the Larmor frequencies of each isotope, yet the Larmor frequency ratio RL  remains constant at the 10 ppm level. We derive the suppression factor η  > 3 × 104   (95% confidence level) based on the two RL  values and their corresponding uncertainties.

Fig. 2  The combined effect of Zeeman and tensor shifts on the Larmor frequency (ω173 ) of the cat state of 173Yb, and on the Larmor frequency ratio RL  = ω171 /ω173 . 173 Yb atoms are  populated  in  |5/2, ±5/2〉g   (green  dots).  (a)  At  ΩB /ΩT   =  2.2, where  |5/2, -5/2〉g   is shifted higher than  |5/2, -3/2〉g, the σx  term mixes the two states in the rectangle and pushes  |5/2, -5/2〉g   further up. This results in a lower frequency ω 173 , and a higher RL value. (b) Under 50 mG, corresponding to ΩB /ΩT  = 2.2, RL  is mapped as the magnetic- field  orientation  is  scanned  around  the  Z-direction.  The  higher  RL   values  in  (a)  results in a surface with positive curvatures.  (c) At ΩB /ΩT  = 4.3, where  |5/2, -5/2〉g   is shifted below  |5/2, -3/2〉g,  the  σx   term  pushes  |5/2, -5/2〉g   further  down.  (d)  Under  100  mG, corresponding to ΩB /ΩT  = 4.3. The curvature of the surface is found to reverse. Error bars: one standard deviation (1-σ) derived from 50 repeated measurements; surfaces: least-squares fittings with quadratic surface functions.

# Residual vector shift

In order to minimize vector shifts, the optical lattice is tuned to be as close to linearly polarized as possible. This is done by optimizing the coherence of the Ramsey interferometry of 171Yb. During polarization tuning, the bias mag- netic field is temporarily redirected to the horizontal propagation direction of the optical lattice (Y-axis), and its strength lowered to 1 mG, both measures

taken to deliberately enhance the decoherence effect due to the inhomoge- neous vector shifts in the lattice. In this way, the residual circular polarization component, as measured by the Stokes parameter S3 , is tuned to be less than 0.1%. According to calculated vector polarizabilities of 171Yb and 173Yb, the vector shifts due to the residue circular polarization can be regarded as effec- tive magnetic fields less than 100 µG along the Y-direction. At a bias field of 50 mG in the Z-direction, the relative deviation in RL  due to the residue vec- tor shift is estimated to be less than 3 × 10-7, which is negligible compared to the current measurement uncertainty of 3 × 10-6 . When the field is increased to 100 mG, the deviation drops further down to 8 × 10-8 .

# Ratio of nuclear magnetic moments

We have investigated systematic uncertainties in the measurements of the Larmor frequency ratio RL . After aligning the 100 mG magnetic field to be parallel to the polarization direction of the optical lattice, we measure RL while varying the lattice power by more than a factor of five. No dependence of RL  on light power is observed within the measurement uncertainties (Fig. 3a). A weighted average across different lattice powers yields RL  = 0.726077(2). Separately, we measure RL while varying the density ratio of 171Yb over 173Yb by a factor of seven. This is to investigate possible collisional shifts that are likely to be density dependent. Again, no dependence of RL  on the density ratio is observed (Fig. 3b). The average value across different density ratios is 0.726072(4). The Earth’s rotation frequency of 2π × 11.6 µHz is negligible compared to the measurement uncertainty of around 2π × 100 µHz for the Larmor frequencies.

Since  the  two  aforementioned  independently  determined  values  of RL agree, we take a weighted average of both, yielding a final value of RL   = 0.726076(3) and the ratio of nuclear magnetic moments between 171Yb and 173Yb, RNMM  = µI,171/µI,173  = -0.726076(3). As shown in Table 1, the new result is consistent with the reported values from prior work  [30–32], while improving the precision by an order of magnitude.

## Table 1  Compilation of reported values.

| Reference | RNMM |
| --- | --- |
| Gossard, A. C. et al., 1964 [30] | -0.7278(16) |
| Olschewski, L., 1972 [31] | -0.72610(4) |
| This work | -0.726076(3) |

# Discussion

With its strong suppression of magnetic-field noise and tight atom localiza- tion within the lattice sites, our cold-atom comagnetometer enables sensitive

Fig.  3  Investigating possible systematic effects.  (a)  In a magnetic field of  100 mG, the lattice beam power is varied. (b) With both the magnetic field and lattice parameters fixed, the density ratio between 171 Yb and 173 Yb is varied. No dependences on these experimental parameters are observed. Error bars: one standard deviation (1-σ) derived from 50 repeated measurements; solid lines: weighted averages.

searches for exotic interactions beyond the Standard Model, particularly at short interaction ranges of 1-100 μm. Among many new-physics interactions proposed [3, 33], we present two examples highlighting the capability of this system.

The comagnetometer is used to search for P- and  T-violating spin-mass couplings, also known as monopole-dipole interactions [12, 13, 19, 34]. In this search, with 171Yb and 173Yb atoms as spin sources, the Larmor frequency ratio is monitored as a dense mass source is brought into proximity with the atoms. By using optical tweezers to position the atoms, the atom-mass separation can be precisely controlled down to a few micrometers. We estimate that this setup can improve the sensitivity to spin-mass couplings at the 1 μm interaction range by up to three orders of magnitude compared to the current experimental limits [35].

Furthermore, we aim to improve the search sensitivity of electric dipole moments (EDMs) using cold atoms [5, 36]. The current EDM experiments on 171Yb [5] are limited by magnetic-field noises, particularly the Johnson noise in adjacent electrodes. A 171Yb-173Yb cold-atom comagnetometer can effectively suppress such noise effects, thereby boosting the EDM search sensitivity.

# Methods

# Zeeman and tensor light shifts of 173Yb

The Hamiltonian in Eq.  (1) is rewritten below, treating the σx   term as a perturbation:

H = H0 + θH′

(4)

The perturbed eigenenergies are

EF)  = mF ΩB + m2FΩT ,                                     (6)

where the θ 2  term corresponds to δEF)   in Eq. (2). When ΩB /ΩT   = 4, the

unperturbed energy levels of |5/2, —5/2〉g  and |5/2, —3/2〉g  are degenerate.

Here the first-order perturbation term is always zero because the σx  term corresponds to a subdiagonal matrix. For the spin-5/2 system in 173Yb, the σx  matrix is

To calculate the Larmor frequency of the cat state of 173Yb, we consider the 2nd-order perturbation on the two populated stretched states |5/2, ±5/2〉g :

Subtracting the two energy shifts yields Eq. (2).

# Calibration of ΩB /ΩT

We perform precession measurements on 173Yb to calibrate the ΩB /ΩT values. Since the cat state is immune to tensor shifts (ΩT ), we employ a coherent spin state (CSS) instead, whose spin is fully polarized along a direction.

While keeping the lattice linearly polarized along the z-axis, its power is reduced to 8 W to mitigate the otherwise rapid decoherence induced by the tensor shift. Meanwhile, a bias magnetic field of 23.5 mG is applied along the x-direction. The  173Yb atoms are first pumped into  |5/2, +5/2〉g , then allowed to precess under the influence of both the Zeeman and tensor shifts,

Hcalib  = ΩB σx + ΩT σ2z . After a variable precession duration (0 — 2,500 ms),

the population signal Sz  is measured along the z-axis (Fig. 4). Since the tensor shift is sufficiently suppressed, it does not yield a state-selective readout.

We simulate the precession process of 173Yb using the calibration Hamilto- nian (Hcalib ) and compare the results with experimental data. Our simulation

models the observed population signal as a sum over all mF  states. The con- tribution of each state is weighted by the Clebsch-Gordan coefficients of the relevant transitions and the detuning of the probe beam. Specifically, when probing  |5/2, +5/2〉g ,  the  detected  signal  comprises  contributions  from  all |5/2, mF〉g  states (mF  = +5/2 to -5/2), with relative weights of 1, 0.58, 0.31, 0.16, 0.08, and 0.03. Furthermore, the spatial distribution of the tensor field is incorporated into the simulation via a Monte Carlo method. The resulting the- oretical curves show good agreement with the experimental data, from which we extract ΩB /ΩT   =  3.5 for the given magnetic field and lattice intensity conditions.

# Atom sample detection

A circularly polarized laser probe beam along the Z-direction runs through the cold atom ensemble and is detected by a CMOS camera. Absorption images are taken on the atoms for population measurements. The state readout of atoms is performed via differential detection of two stretched states. For 171Yb atoms,

t|h32,o3b/e2〉bee(amFig .re5soa)nantMlyeanexwhciiltee,stthhee|1cy/2c,l-ing1/t2angtio|2|,121,/+2〉1e/2tgan-

tion is off resonance by 10 times the natural linewidth (10 Γ) due to tensor shifts on the excited states. As a result, the cycling transition can be repeated 1,200 times on average before a spin flip occurs. This spin selectivity ensures that  the  spin  state  is  detected  with  an  efficiency  approaching  100%.  For

of tensor shifts (    20 Γ), the branching ratios of these two transitions differ by a factor of 21, thus further enhancing the spin selectivity.

1|75Y2,b- a5t/o2m〉gsh|r2e,lv3n2t〉etr(ansFigitio5bn)s  aInread|5/2ditio,+n5t2a〉gcp| 7a2ab,l+7/e di2ffreanncde

# Acknowledgements

We  thank  D.   Sheng  for  fruitful  discussions  and   critical  reading  of  the manuscript, and thank S.-Z. Wang for contributions to the apparatus during the early stages.

# Declarations

• Funding

This work  is  supported  by the  National  Natural  Science  Foundation  of China  (NSFC) through  grant no.  12174371, the Innovation  Program for Quantum Science and Technology through grant no. 2021ZD0303101, the Strategic Priority Research Program of the Chinese Academy of Sciences through grant no. XDB21010200, and the Major Frontier Research Project of the University of Science and Technology of China through grant no. LS9990000002.

• Author contribution

1

0.5

Sz

0

-0.5

-1

0            500         1000        1500        2000        2500 Precession ms

Fig. 4   Precession of a coherent spin state of 173 Yb under Bx  = 23.5 mG and 8 W lattice power. Error bars: one standard deviation (1-σ) derived from 3 repeated measurements. The solid curve represents simulation results.

a

b

1

1/3

1

1/21

Fig.  5    Transitions used to measure spin-state populations in  171Yb  (a) and  173Yb  (b). The arrow of solid line indicates a cycling transition used for population measurements; the arrow of dashed line indicates a spin-flip transition suppressed by large detunings.

J.-L. Z. and W.-T. L. constructed the experimental apparatus and per- formed  the  experiments  and  simulations.  Y.  A.  Y.  contributed  to  the apparatus during the early stages. Y.-Q. W. made contributions to the the- ory part. J.-L. Z., W.-T. L., Y.-Q. W., T. X. and Z.-T. L. carried out data analysis and wrote the manuscript.

# References

[1]  Pezz`e, L., Smerzi, A., Oberthaler, M. K., Schmied, R. & Treutlein, P. Quantum metrology with nonclassical states of atomic ensembles.  Rev. Mod. Phys. 90, 035005 (2018).  URL https://link.aps.org/doi/10.1103/ RevModPhys.90.035005.

[2] Budker, D. & Romalis, M. Optical magnetometry. Nat. Phys. 3, 227–234 (2007). URL http://dx.doi.org/10.1038/nphys566.

[3]  Cong, L. et al.  Spin-dependent exotic interactions.  Rev. Mod. Phys. 97, 025005 (2025).  URL https://link.aps.org/doi/10.1103/RevModPhys.97. 025005.

[4] Kominis, I. K., Kornack, T. W., Allred, J. C. & Romalis, M. V. A subfem- totesla multichannel atomic magnetometer.  Nature 422, 596–599 (2003). URL http://dx.doi.org/10.1038/nature01484.

[5]  Zheng, T. A. et al. Measurement of the electric dipole moment of 171Yb atoms in an optical dipole trap.   Phys. Rev. Lett. 129, 083001  (2022). URL https://link.aps.org/doi/10.1103/PhysRevLett.129.083001.

[6] Terrano, W. A. & Romalis, M. V. Comagnetometer probes of dark matter and new physics.  Quantum Sci.  Technol. 7, 014001 (2021).  URL https: //dx.doi.org/10.1088/2058-9565/ac1ae0.

[7] Vasilakis, G., Brown, J. M., Kornack, T. W. & Romalis, M. V.  Lim- its on new long range nuclear spin-dependent forces set with a K-3 He comagnetometer.    Phys.  Rev.  Lett.  103,  261801  (2009).    URL https:

//link.aps.org/doi/10.1103/PhysRevLett.103.261801.

[8] Kimball, D. F. J.  et  al.   A  dual-isotope  rubidium  comagnetometer to search for anomalous long-range spin-mass (spin-gravity) couplings of the proton.  Ann.  Phys.  (Berlin) 525, 514–528 (2013).  URL http://dx.doi. org/10.1002/andp.201300036.

[9] Almasi, A., Lee, J., Winarto, H., Smiciklas, M. & Romalis, M. V.  New limits on anomalous spin-spin interactions. Phys. Rev. Lett. 125, 201802 (2020). URL https://link.aps.org/doi/10.1103/PhysRevLett.125.201802.

[10]  Can`e, F. et al. Bound on Lorentz and CPT violating boost effects for the neutron. Phys. Rev. Lett. 93, 230801 (2004). URL https://link.aps.org/ doi/10.1103/PhysRevLett.93.230801.

[11] Allmendinger,  F.  et  al.    New  limit  on  Lorentz-invariance-  and  CPT- violating neutron spin interactions using a free-spin-precession 3 He-129Xe comagnetometer.    Phys.  Rev.  Lett.  112,  110801  (2014).    URL https: //link.aps.org/doi/10.1103/PhysRevLett.112.110801.

[12] Bulatowicz, M. et al.  Laboratory search for a long-range T-odd, P-odd interaction from axionlike particles using dual-species nuclear magnetic resonance with polarized  129Xe and  131Xe gas.   Phys.  Rev.  Lett.  111, 102001 (2013). URL https://link.aps.org/doi/10.1103/PhysRevLett.111. 102001.

[13] Feng, Y.-K., Ning, D.-H., Zhang, S.-B., Lu, Z.-T. & Sheng, D.  Search for  monopole-dipole  interactions  at  the  submillimeter  range  with  a 129Xe-131Xe-Rb comagnetometer. Phys. Rev. Lett. 128, 231803 (2022). URL https://link.aps.org/doi/10.1103/PhysRevLett.128.231803.

[14]  Lamoreaux, S. K., Jacobs, J. P., Heckel, B. R., Raab, F. J. & Fortson, E. N. New limits on spatial anisotropy from optically-pumped 201 Hg and 199 Hg. Phys. Rev. Lett. 57, 3125–3128 (1986). URL https://link.aps.org/ doi/10.1103/PhysRevLett.57.3125.

[15] Kornack, T. W., Ghosh, R. K. & Romalis, M. V. Nuclear spin gyroscope based on an atomic comagnetometer. Phys. Rev. Lett. 95, 230801 (2005). URL https://link.aps.org/doi/10.1103/PhysRevLett.95.230801.

[16] Rosenberry,  M.  A.  &  Chupp,  T.  E.    Atomic  electric  dipole  moment measurement  using  spin  exchange  pumped  masers  of  129Xe  and  3 He. Phys.  Rev.  Lett.  86,  22–25  (2001).    URL  http://dx.doi.org/10.1103/ PhysRevLett.86.22.

[17]  Sachdeva, N. et al.  New limit on the permanent electric dipole moment of  129Xe  using  3 He  comagnetometry  and   SQUID  detection.      Phys.

Rev. Lett. 123, 143003 (2019).  URL https://link.aps.org/doi/10.1103/ PhysRevLett.123.143003.

[18] Abel, C. et al. Measurement of the permanent electric dipole moment of the neutron. Phys. Rev. Lett. 124, 081803 (2020). URL https://link.aps. org/doi/10.1103/PhysRevLett.124.081803.

[19] Tullney, K. et al. Constraints on spin-dependent short-range interaction between nucleons.   Phys.  Rev.  Lett.  111,  100801  (2013).    URL https: //link.aps.org/doi/10.1103/PhysRevLett.111.100801.

[20] Venema, B. J., Majumder, P. K., Lamoreaux, S. K., Heckel, B. R. & Fortson, E. N.  Search for a coupling of the earth’s gravitational field to nuclear spins in atomic mercury.  Phys. Rev. Lett. 68, 135–138 (1992). URL https://link.aps.org/doi/10.1103/PhysRevLett.68.135.

[21]  Zhang, S.-B. et al.  Search for spin-dependent gravitational interactions at earth range.  Phys. Rev. Lett. 130, 201401 (2023).  URL https://link. aps.org/doi/10.1103/PhysRevLett.130.201401.

[22] Wu, T. et al. Search for axionlike dark matter with a liquid-state nuclear spin comagnetometer.  Phys. Rev. Lett. 122, 191302 (2019).  URL https: //link.aps.org/doi/10.1103/PhysRevLett.122.191302.

[23] Alonso, R., Blas, D. & Wolf, P. Exploring the ultra-light to sub-Mev dark matter window with atomic clocks and co-magnetometers. J. High Energy Phys. 2019 (2019). URL http://dx.doi.org/10.1007/JHEP07(2019)069.

[24]  Gomez,  P.  et  al.     Bose-Einstein  Condensate  comagnetometer.    Phys. Rev. Lett. 124, 170401 (2020).  URL https://link.aps.org/doi/10.1103/ PhysRevLett.124.170401.

[25] Thekkeppatt, P., Digvijay, Urech, A., Schreck, F. & van Druten, K. Mea- surement of the g factor of ground-state 87 Sr at the parts-per-million level using co-trapped ultracold atoms.  Phys.  Rev. Lett. 135, 193001 (2025). URL https://link.aps.org/doi/10.1103/cjks-9hlp.

[26]  Zheng, T. A. et al.  Magic wavelengths of the Yb (6s2  1 S0  - -6s6p 3 P1 ) intercombination transition.   Phys.  Rev.  A  102,  062805  (2020).   URL https://link.aps.org/doi/10.1103/PhysRevA.102.062805.

[27] Yang,  Y.  A.   et   al.     Minute-scale   Schr¨odinger-cat  state  of  spin-5/2 atoms. Nat. Photon. 19, 89–94 (2024). URL http://dx.doi.org/10.1038/ s41566-024-01555-3.

[28]  Le Kien, F., Schneeweiss, P. & Rauschenbeutel, A.  Dynamical polariz- ability of atoms in arbitrary light fields: general theory and application

to cesium. Eur. Phys. J. D 67 (2013). URL http://dx.doi.org/10.1140/ epjd/e2013-30729-x.

[29] Budker, D., Kimball, D. F., Rochester, S. M. & Yashchuk, V. V. Nonlinear magneto-optical rotation via alignment-to-orientation conversion.  Phys. Rev. Lett. 85, 2088–2091 (2000). URL https://link.aps.org/doi/10.1103/ PhysRevLett.85.2088.

[30]  Gossard, A. C., Jaccarino, V. & Wernick, J. H.  Ytterbium NMR:Yb171 nuclear moment and Yb metal knight shift.  Phys. Rev. 133, A881–A884 (1964). URL https://link.aps.org/doi/10.1103/PhysRev.133.A881.

[31]  Olschewski, L.  Messung der magnetischen kerndipolmomente an freien 43 Ca-,  87 Sr-,  135 Ba-,  137 Ba-,  171Yb-  und173Yb-Atomen  mit  optischem pumpen. Z. Phys. 249, 205–227 (1972). URL http://dx.doi.org/10.1007/ BF01400226.

[32]  Stone,  N.    Table  of  nuclear  magnetic  dipole  and  electric  quadrupole moments.   At.  Data Nucl. Data  Tables  90,  75–176  (2005).   URL http: //dx.doi.org/10.1016/j.adt.2005.04.001.

[33]  Safronova, M. S. et al. Search for new physics with atoms and molecules. Rev. Mod. Phys. 90, 025008 (2018).  URL https://link.aps.org/doi/10. 1103/RevModPhys.90.025008.

[34]  Moody,  J. E. & Wilczek, F.   New  macroscopic  forces?   Phys.  Rev. D 30, 130–138 (1984).  URL https://link.aps.org/doi/10.1103/PhysRevD. 30.130.

[35]  Guigue, M., Jullien, D., Petukhov, A. K. & Pignol, G. Constraining short- range spin-dependent forces with polarized 3 He. Phys. Rev. D 92, 114001 (2015). URL https://link.aps.org/doi/10.1103/PhysRevD.92.114001.

[36] Bishof, M. et al.  Improved limit on the 225 Ra electric dipole moment. Phys. Rev. C 94, 025501 (2016). URL https://link.aps.org/doi/10.1103/ PhysRevC.94.025501.